# Chapter 1: Introduction – The Big Picture

## What is the Internet?

### 🥜 "Nuts and Bolts" View

The Internet is a network of networks connecting billions of devices.

| Component | Description |
| --- | --- |
| Hosts / End systems | Devices running network apps (PCs, servers, smartphones, IoT, toasters with weather forecasters!) |
| Packet switches | Routers and switches that forward chunks of data (packets) |
| Communication links | Fiber, copper, radio, satellite – with transmission rates (bandwidth) |
| ISPs | Internet Service Providers (home, enterprise, mobile, content provider networks) |

Protocols everywhere! HTTP, TCP, IP, WiFi, 4G/5G, Ethernet.

Standards bodies:

- RFC – Request for Comments (technical specifications)

- IETF – Internet Engineering Task Force (develops RFCs)

### 🎁 "Services" View

The Internet is an infrastructure that provides services to applications:

- Web, streaming video, email, games, social media, Zoom, etc.

- Provides a programming interface (socket API) for apps to use the transport service.

## What is a Protocol?

A protocol defines the format, order of messages sent/received, and actions taken on message transmission or receipt.

Human analogy:

- "Hi" → "Hi" (greeting)

- "Got the time?" → "2:00" (request-response)

Network protocol example (HTTP):

text

Client: GET /index.html HTTP/1.1

Server: HTTP/1.1 200 OK (followed by HTML data)

## Network Edge: Hosts, Access Networks, Physical Media

### 🖥️ Hosts (End Systems)

- Clients – request services (desktop, mobile)

- Servers – provide services (often in data centers)

### 📡 Access Networks – How hosts connect to edge router

| Type | Technology | Speeds | Characteristics |
| --- | --- | --- | --- |
| Cable-based | HFC (Hybrid Fiber Coax) | Down: 40 Mbps – 1.2 Gbps, Up: 30-100 Mbps | Shared, asymmetric, FDM |
| DSL | Digital Subscriber Line | Down: 24-52 Mbps, Up: 3.5-16 Mbps | Dedicated, uses phone line |
| Home network | WiFi + Ethernet + DSL/cable modem | 100 Mbps – 1 Gbps | Mix of wired/wireless |
| Wireless LAN (WiFi) | 802.11 a/b/g/n/ac/ax | 11 – 1000+ Mbps | Within ~100 ft, shared |
| Cellular | 4G/5G | 10's – 100's Mbps | Wide-area (10's km) |
| Enterprise | Ethernet + WiFi | 100 Mbps – 10 Gbps | Switches, routers, high bandwidth |
| Data center | High-speed Ethernet | 10's – 100's Gbps | Thousands of servers |

### 🔌 Physical Media

| Medium | Characteristics | Speed |
| --- | --- | --- |
| Twisted pair (Cat5/6) | Copper, cheap, common | 100 Mbps – 10 Gbps |
| Coaxial cable | Copper, bidirectional, broadband | 100's Mbps per channel |
| Fiber optic | Glass, light pulses, low error, immune to noise | 10's – 100's Gbps |
| Wireless radio | No physical wire, half-duplex, interference | 10's – 100's Mbps (WiFi), 10's Mbps (4G) |

### 📦 Host: Sending Packets

- Takes application message, breaks into packets of length L bits

- Transmits packet into access network at transmission rate R (link bandwidth)

- Packet transmission delay = L / R seconds

## Network Core: Packet Switching, Circuit Switching

### 🔄 Packet Switching

- Hosts break messages into packets

- Packets forwarded from router to router

- Store-and-forward: entire packet must arrive before being transmitted on next link

Transmission delay = L (bits) / R (bits/sec)

Queueing: Packets wait in buffers if arrival rate > transmission rate → loss if buffer full.

### 🔌 Circuit Switching (Traditional Telephone)

- End-end resources reserved for a "call" (dedicated circuits)

- No sharing – circuit idle if not used

- FDM (Frequency Division Multiplexing): frequency bands allocated per call

- TDM (Time Division Multiplexing): time slots allocated per call

### ⚖️ Packet Switching vs Circuit Switching

| Feature | Packet Switching | Circuit Switching |
| --- | --- | --- |
| Resource sharing | Statistical multiplexing | Dedicated allocation |
| Call setup | Not required | Required |
| Congestion | Possible (delay/loss) | None (guaranteed) |
| Efficiency | High for bursty data | Low for bursty data |
| Example | Internet | Old telephone network |

Classic example: 1 Gb/s link, each user active 10% of time at 100 Mb/s.

- Circuit-switching: max 10 users

- Packet-switching: with 35 users, probability >10 active is <0.0004 → much more efficient!

### Two Key Network-Core Functions

| Function | Type | Description |
| --- | --- | --- |
| Forwarding | Local | Move arriving packets from input link to appropriate output link (aka switching) |
| Routing | Global | Determine source-destination paths taken by packets (using routing algorithms) |

## Internet Structure: Network of Networks

text

Access ISPs → Regional ISPs → Tier-1 ISPs ↔ Content Provider Networks

- IXP (Internet Exchange Point): connects multiple ISPs

- Peering – direct connection between ISPs (no payment)

- Tier-1 ISPs – large national/international (Level 3, AT&T, NTT)

- Content provider networks – Google, Facebook run own private networks to bring content closer to users.

Evolution driven by economics and national policies.

## Performance: Delay, Loss, Throughput

### ⏱️ Four Sources of Packet Delay

Total nodal delay = d_proc + d_queue + d_trans + d_prop

| Delay Type | Cause | Magnitude |
| --- | --- | --- |
| d_proc (nodal processing) | Check bit errors, determine output link | < microseconds |
| d_queue (queueing) | Waiting at output link for transmission | Depends on congestion (can be large) |
| d_trans (transmission) | Push bits onto link: L/R | Microseconds to milliseconds |
| d_prop (propagation) | Travel over physical medium: distance / speed of light | Milliseconds |

### 🚗 Caravan Analogy

- Car ~ bit; caravan ~ packet; toll service ~ link transmission

- Toll booth takes 12 sec to service car (bit transmission time), propagate at 100 km/hr

- Q: How long until caravan lined up before 2nd toll booth?

- Time to push entire caravan through = 12×10 = 120 sec

- Time for last car to propagate 100 km = 1 hr

- A: 62 minutes

If propagation speed increases to 1000 km/hr and toll time = 1 min, first car arrives at 2nd booth in 7 min while 3 cars still at first booth.

### 📊 Queueing Delay (revisited)

- a: average packet arrival rate

- L: packet length (bits)

- R: link bandwidth (bps)

- Traffic intensity = La / R

  - La/R ~ 0 → small avg queueing delay

  - La/R → 1 → large avg queueing delay

  - La/R > 1 → more work arriving than can be serviced → average delay infinite!

### 🔍 Traceroute

- Provides delay measurement from source to each router along path

- Sends 3 packets with increasing TTL (Time-To-Live)

- Each router returns a probe, sender measures RTT

- * means no response (probe lost, router not replying)

Sample output (gaia.cs.umass.edu to www.eurecom.fr):

text

1 cs-gw (128.119.240.254) 1 ms 1 ms 2 ms

2 border1-rt... (128.119.3.145) 1 ms 1 ms 2 ms

...

15 eurecom-valbonne... 135 ms 128 ms 133 ms

19 fantasia.eurecom.fr 132 ms 128 ms 136 ms

### 📉 Packet Loss

- Queue (buffer) preceding link has finite capacity

- Packet arriving to full queue → dropped (lost)

- Lost packet may be retransmitted by previous node, by source, or not at all.

### 📈 Throughput

- Throughput = rate (bits/time) at which bits are sent from sender to receiver

- Instantaneous – rate at given point in time

- Average – rate over longer period

- Bottleneck link – the link on the end-end path with the smallest transmission rate

- Per-connection throughput ≈ min(Rc, Rs, R/10) in shared scenarios

## Protocol Layers & Encapsulation

### 🧅 Why Layering?

- Explicit structure identifies relationship of system's pieces

- Modularization eases maintenance and updating

- Change in layer implementation is transparent to rest of system

### Internet Protocol Stack (5 layers)

| Layer | Unit | Function | Example Protocols |
| --- | --- | --- | --- |
| Application | Message | Network apps | HTTP, SMTP, DNS |
| Transport | Segment | Process-to-process data transfer | TCP, UDP |
| Network | Datagram | Host-to-host routing | IP, routing protocols |
| Link | Frame | Data transfer between neighboring network elements | Ethernet, WiFi, PPP |
| Physical | Bit | Raw bits on wire | Fiber, copper, radio |

### ISO/OSI Reference Model (7 layers)

Adds two layers above application:

- Presentation – encryption, compression, machine-specific conventions

- Session – synchronization, checkpointing, recovery

Internet stack doesn't have these – they're implemented in application if needed.

### Encapsulation (like Matryoshka dolls)

text

Application:    Message M

Transport:      [Ht | M]  → Segment

Network:        [Hn | Ht | M] → Datagram

Link:           [Hl | Hn | Ht | M] → Frame

Each layer adds its own header (control information).

## Network Security

Original Internet: "mutually trusting users" – not designed with much security.

### 🕵️ Bad Guys at Work

| Attack | Description |
| --- | --- |
| Packet sniffing | Promiscuous interface reads all passing packets (e.g., Wireshark) |
| IP spoofing | Inject packets with false source address |
| Denial of Service (DoS) | Overwhelm resource with bogus traffic (often via botnet) |

### 🛡️ Lines of Defense

- Authentication – proving identity (passwords, certificates)

- Confidentiality – encryption (TLS)

- Integrity checks – digital signatures

- Access restrictions – VPNs, firewalls

- Detecting DoS – specialized middleboxes

## History of the Internet

| Era | Milestones |
| --- | --- |
| 1961-1972 | Packet-switching principles (Kleinrock), ARPANet (1969), first email (1972) |
| 1972-1980 | Internetworking (Cerf & Kahn, 1974), Ethernet (1976), proprietary nets |
| 1980-1990 | TCP/IP deployment (1983), DNS (1983), SMTP (1982), congestion control (1988) |
| 1990-2000s | Web (HTML, HTTP, Mosaic), commercialization, killer apps (IM, P2P) |
| 2005-present | Broadband home access, SDN (2008), 4G/5G/WiFi, cloud, content provider networks |

Key Internetworking Principles (Cerf & Kahn):

- Minimalism, autonomy (no internal changes to interconnect networks)

- Best-effort service model

- Stateless routing

- Decentralized control

# Chapter 2: Application Layer

## Principles of Network Applications

### Process Communication

- Process = program running on a host.

- Client process – initiates communication.

- Server process – waits to be contacted.

- P2P architectures – both client and server roles.

### Socket – The Doorway

text

Application ←→ Socket ←→ Transport layer ←→ Network

- Socket = software interface between application and transport.

- Sending process shoves message out the "door", transport delivers to receiving socket.

### Addressing Processes

- IP address (32-bit) identifies host.

- Port number identifies specific process on that host.

- Example: HTTP server → port 80; mail server → port 25.

### Application Architectures

| Architecture | Description | Examples |
| --- | --- | --- |
| Client-Server | Server always-on, permanent IP, clients contact server | HTTP, IMAP, FTP |
| Peer-to-Peer (P2P) | No always-on server, arbitrary end systems communicate, self-scalability | BitTorrent, Skype (hybrid) |

### What Transport Service Does an App Need?

| Requirement | Description | Example Apps |
| --- | --- | --- |
| Data integrity | 100% reliable transfer | File transfer, web, email |
| Timing | Low delay critical | Internet telephony, games |
| Throughput | Minimum required for effectiveness | Multimedia (elastic apps use whatever) |
| Security | Encryption, data integrity | Banking, VPN |

### Internet Transport Protocols: TCP vs UDP

| Feature | TCP | UDP |
| --- | --- | --- |
| Reliability | ✅ Reliable | ❌ Unreliable |
| Flow control | ✅ | ❌ |
| Congestion control | ✅ | ❌ |
| Connection-oriented | ✅ (handshake) | ❌ |
| Timing guarantee | ❌ | ❌ |
| Throughput guarantee | ❌ | ❌ |
| Security | ❌ (but TLS adds it) | ❌ |

Why UDP? Lower overhead, no delay for connection setup, fine for loss-tolerant apps (voice, video, DNS).

### Securing TCP: TLS (Transport Layer Security)

- Provides encrypted TCP connections, data integrity, end-point authentication

- Implemented in application layer (apps use TLS libraries that use TCP)

- Cleartext sent into socket → traverses Internet encrypted

## Web & HTTP

### HTTP (Hypertext Transfer Protocol)

- Web's application-layer protocol.

- Client-server model: browser requests, server responds.

- Uses TCP port 80 (or 443 for HTTPS).

- Stateless – server maintains no information about past client requests.

Statelessness simplifies server design but complicates stateful interactions (shopping carts). Solution? Cookies!

### HTTP Connections

| Type | Behavior | RTT per object |
| --- | --- | --- |
| Non-persistent (HTTP/1.0) | One TCP connection per object | 2 RTT + transmission |
| Persistent (HTTP/1.1) | Single connection for multiple objects | 1 RTT for all |

Non-persistent response time: 2 RTT + file transmission time.

- 1 RTT to establish TCP connection

- 1 RTT for request + response headers

Persistent HTTP: Server leaves connection open; client sends requests as soon as it encounters referenced objects.

### HTTP Request Message (ASCII format)

text

GET /somedir/page.html HTTP/1.1

Host: www.someschool.edu

User-Agent: Mozilla/5.0

Accept: text/html,application/xhtml+xml

Accept-Language: en-us

Connection: keep-alive

(empty line)

Request line format: Method sp URL sp Version cr lf

### HTTP Response Message

text

HTTP/1.1 200 OK

Connection: close

Date: Tue, 18 Aug 2023 15:00:00 GMT

Server: Apache/2.2.3

Last-Modified: Tue, 18 Aug 2023 14:00:00 GMT

Content-Length: 6821

Content-Type: text/html

(data data data ...)

### HTTP Status Codes

| Code | Meaning |
| --- | --- |
| 200 OK | Request succeeded |
| 301 Moved Permanently | Object moved, new location given |
| 400 Bad Request | Request message not understood |
| 404 Not Found | Document not found |
| 505 HTTP Version Not Supported |  |

### HTTP Methods

| Method | CRUD Analogy | Description |
| --- | --- | --- |
| GET | Read | Request resource; data in URL |
| POST | Create | Submit form input (entity body) |
| HEAD | (metadata) | Requests headers only (no object) |
| PUT | Update | Upload file to server (replaces existing) |

Example GET with data: www.somesite.com/animalsearch?monkeys&banana

## HTTP Caching (Web Caches)

### How Web Cache Works

- User configures browser to point to local web cache.

- Browser sends all HTTP requests to cache.

- If object in cache → cache returns object to client.

- Else → cache requests object from origin server, caches it, then returns to client.

### Why Web Caching?

- Reduce response time (cache is closer)

- Reduce traffic on institution's access link

- Enable "poor" content providers to deliver content more effectively

### Example: Access Link Performance

Scenario:

- Access link rate: 1.54 Mbps

- RTT to origin server: 2 sec

- Object size: 100 Kbits

- Request rate: 15/sec → avg data rate 1.50 Mbps

Without cache:

- Access link utilization = 0.97 (97%) → large queueing delays (minutes!)

With cache (hit rate = 0.4):

- Rate to browsers over access link = 0.6 × 1.50 = 0.9 Mbps

- Access link utilization = 0.9/1.54 = 0.58 → low queueing delay

- Average end-end delay = 0.6×2.01 + 0.4×(msec) ≈ 1.2 sec (better than faster link!)

### Conditional GET

- Prevents sending object if cache has up-to-date version.

- Client includes: If-modified-since: <date>

- Server responds with 304 Not Modified if cached copy is up-to-date (no object transmission).

## HTTP/2 & HTTP/3

### HTTP/1.1 Problems

- Head-of-line (HOL) blocking: small object waits behind large object(s) due to FCFS scheduling.

- Loss recovery stalls object transmission.

### HTTP/2 [RFC 7540, 2015]

- Increased flexibility at server in sending objects:

  - Transmission order based on client-specified priority (not FCFS)

  - Push unrequested objects to client

  - Divide objects into frames, schedule frames to mitigate HOL blocking

Example: Client requests 1 large video + 3 small objects. HTTP/2 interleaves frames → small objects delivered quickly.

### HTTP/3

- HTTP/2 still over single TCP connection → recovery from packet loss still stalls all transmissions.

- HTTP/3: adds security, per-object error- and congestion-control over UDP (more pipelining).

- Used in modern web.

## Email: SMTP, POP3, IMAP

### Three Major Components

- User agents (mail readers: Outlook, iPhone mail client)

- Mail servers (mailbox for incoming, message queue for outgoing)

- SMTP (Simple Mail Transfer Protocol)

### SMTP [RFC 5321]

- Uses TCP, port 25

- Direct transfer: sending mail server (acting as client) to receiving mail server

- Three phases: handshake (greeting), transfer of messages, closure

- Command/response interaction (ASCII)

- Persistent connections

- Requires message (header & body) in 7-bit ASCII

- Uses CRLF.CRLF to determine end of message

### Sample SMTP Interaction

text

S: 220 hamburger.edu

C: HELO crepes.fr

S: 250 Hello crepes.fr, pleased to meet you

C: MAIL FROM: <alice@crepes.fr>

S: 250 alice@crepes.fr... Sender ok

C: RCPT TO: <bob@hamburger.edu>

S: 250 bob@hamburger.edu... Recipient ok

C: DATA

S: 354 Enter mail, end with "." on a line by itself

C: Do you like ketchup?

C: How about pickles?

C: .

S: 250 Message accepted for delivery

C: QUIT

S: 221 closing connection

### Mail Message Format [RFC 2822]

text

header: To: bob@hamburger.edu

From: alice@crepes.fr

Subject: Lunch?

(blank line)

body: Do you like ketchup? How about pickles?

### Mail Access Protocols (Retrieving Email)

| Protocol | Description |
| --- | --- |
| POP3 (Post Office Protocol) | Downloads messages to client (usually deletes from server) |
| IMAP (Internet Mail Access Protocol) | Messages stored on server; provides retrieval, deletion, folders |
| HTTP | Web-based interface (Gmail, Hotmail, etc.) on top of SMTP (send) and IMAP/POP (retrieve) |

## DNS (Domain Name System)

### Why DNS?

- Humans use names (e.g., au.kamra.edu)

- Routers use IP addresses (32-bit)

- DNS maps between names and IP addresses.

### DNS Services

- Hostname-to-IP-address translation

- Host aliasing (canonical vs alias names)

- Mail server aliasing

- Load distribution (replicated web servers: many IPs → one name)

### Why Not Centralize DNS?

- Single point of failure

- Traffic volume (Comcast: 600B queries/day; Akamai: 2.2T queries/day!)

- Distant centralized database (high delay)

- Maintenance

### DNS: Distributed, Hierarchical Database

text

Root DNS servers (13 logical, many replicated)

↓

Top-Level Domain (TLD) servers (.com, .org, .edu, .pk, .cn, ...)

↓

Authoritative DNS servers (organization's own DNS servers)

↓

Local DNS name server (your ISP's DNS, not strictly in hierarchy)

### DNS Name Resolution

Iterated query (client contacts server, server replies with next server to contact):

text

Host at engineering.nyu.edu wants IP of gaia.cs.umass.edu

1. Query root server → reply: ask .edu TLD

2. Query .edu TLD → reply: ask umass.edu authoritative

3. Query umass.edu authoritative → reply: IP address

Recursive query (puts burden on contacted name server):

- Heavy load at upper levels of hierarchy.

### DNS Caching

- Once any name server learns a mapping, it caches it

- Improves response time

- Cache entries timeout after TTL (Time To Live)

- Cached entries may be out-of-date (best-effort)

### DNS Resource Records (RR)

Format: (name, value, type, ttl)

| Type | Name | Value | Meaning |
| --- | --- | --- | --- |
| A | hostname | IP address | Hostname-to-IP |
| NS | domain (e.g., foo.com) | hostname of authoritative name server |  |
| CNAME | alias name | canonical (real) name | Alias-to-real mapping |
| MX | domain | name of SMTP mail server | Mail exchange |

### DNS Protocol Messages (Query & Reply)

Same format:

- Header: identification (16-bit), flags (query/reply, recursion desired, recursion available, authoritative)

- Question: name, type fields

- Answer: RRs in response

- Authority: records for authoritative servers

- Additional: "helpful" info

### Getting Your Info into DNS

- Register domain name at DNS registrar (e.g., Network Solutions)

- Provide names, IP addresses of authoritative name servers (primary & secondary)

- Registrar inserts NS, A RRs into TLD server

- Create authoritative server locally with type A record for www.networkutopia.com and type MX record

### DNS Security

- DDoS attacks on root servers → traffic filtering, caching help mitigate

- Spoofing attacks (intercept DNS queries, return bogus replies)

- DNS cache poisoning

- DNSSEC provides authentication, message integrity

## Peer-to-Peer (P2P) & BitTorrent

### P2P Architecture

- No always-on server

- Arbitrary end systems (peers) directly communicate

- Self scalability – new peers bring new service capacity and new demands

- Peers are intermittently connected, change IP addresses

- Complex management

### File Distribution: Client-Server vs P2P

Client-server time to distribute file of size F to N clients:

text

D_c-s = max{ N·F/u_s , F/d_min }

where u_s = server upload capacity, d_min = min client download rate.

P2P time:

text

D_P2P = max{ F/u_s , F/d_min , N·F/(u_s + Σ u_i) }

### BitTorrent

- File divided into 256Kb chunks

- Tracker tracks peers participating in torrent (group exchanging chunks)

- Tit-for-tat: Alice sends chunks to the four peers currently sending her chunks at highest rate (others choked). Re-evaluate every 10 sec.

- Optimistically unchoke one random peer every 30 sec (to discover better partners)

- Rarest first – request missing chunks that are rarest among neighbors

## Content Distribution Networks (CDNs)

### How Netflix Works

- Subscriber requests content

- Service provider returns manifest

- Client retrieves content from OpenConnect CDN nodes (worldwide)

- May choose different rate or copy if network path congested

### CDN Challenges (OTT – "Over the Top")

- What content to place in which CDN node?

- From which CDN node to retrieve content? At which rate?

- Coping with congested Internet from the "edge"

## Socket Programming (UDP & TCP)

### Socket: Door Between Application and Transport

- Two socket types: UDP (unreliable datagram) and TCP (reliable, byte-stream-oriented)

### UDP Socket Programming (Python)

UDP Client:

python

from socket import *

serverName = 'hostname'

serverPort = 12000

clientSocket = socket(AF_INET, SOCK_DGRAM)

message = input('Input lowercase sentence:')

clientSocket.sendto(message.encode(), (serverName, serverPort))

modifiedMessage, serverAddress = clientSocket.recvfrom(2048)

print(modifiedMessage.decode())

clientSocket.close()

UDP Server:

python

from socket import *

serverPort = 12000

serverSocket = socket(AF_INET, SOCK_DGRAM)

serverSocket.bind(('', serverPort))

print('The server is ready to receive')

while True:

message, clientAddress = serverSocket.recvfrom(2048)

modifiedMessage = message.decode().upper()

serverSocket.sendto(modifiedMessage.encode(), clientAddress)

### TCP Socket Programming (Python)

TCP Client:

python

from socket import *

serverName = 'servername'

serverPort = 12000

clientSocket = socket(AF_INET, SOCK_STREAM)

clientSocket.connect((serverName, serverPort))

sentence = input('Input lowercase sentence:')

clientSocket.send(sentence.encode())

modifiedSentence = clientSocket.recv(1024)

print('From Server:', modifiedSentence.decode())

clientSocket.close()

TCP Server:

python

from socket import *

serverPort = 12000

serverSocket = socket(AF_INET, SOCK_STREAM)

serverSocket.bind(('', serverPort))

serverSocket.listen(1)

print('The server is ready to receive')

while True:

connectionSocket, addr = serverSocket.accept()

sentence = connectionSocket.recv(1024).decode()

capitalizedSentence = sentence.upper()

connectionSocket.send(capitalizedSentence.encode())

connectionSocket.close()

# Chapter 3: Transport Layer

## Transport Layer Services & Multiplexing

### Transport vs Network Layer

| Layer | Communication Between |
| --- | --- |
| Network layer | Hosts |
| Transport layer | Processes (relying on, enhancing network layer services) |

### Transport Layer Actions

Sender:

- Is passed application-layer message

- Determines segment header fields, creates segment

- Passes segment to IP

Receiver:

- Receives segment from IP

- Checks header values, extracts application-layer message

- Demultiplexes message up to application via socket

### Two Principal Internet Transport Protocols

| Protocol | Features |
| --- | --- |
| TCP | Reliable, in-order delivery; congestion control; flow control; connection setup |
| UDP | Unreliable, unordered delivery; no-frills extension of "best-effort" IP |

Services not available: delay guarantees, bandwidth guarantees.

### Multiplexing/Demultiplexing

- Multiplexing at sender: gather data from multiple sockets, encapsulate with header, send

- Demultiplexing at receiver: use header info to deliver received segments to correct socket

### Connectionless Demultiplexing (UDP)

- Socket identified by destination port number only

- UDP socket created with DatagramSocket(port#)

- Different source IP/port → same socket at receiving host

### Connection-Oriented Demultiplexing (TCP)

- TCP socket identified by 4-tuple: (source IP, source port, dest IP, dest port)

- Server may support many simultaneous TCP sockets, each with its own 4-tuple

## UDP (User Datagram Protocol)

### "No frills," "bare bones"

- "Best effort" service: segments may be lost, delivered out-of-order

- Connectionless: no handshaking

- Each UDP segment handled independently

### UDP Use Cases

- Streaming multimedia apps (loss tolerant, rate sensitive)

- DNS, SNMP

- HTTP/3 (reliability added at application layer)

### UDP Segment Header

text

0      7 8     15 16    23 24    31

+--------+--------+--------+--------+

|     Source Port      |  Dest Port     |

+--------+--------+--------+--------+

|        Length       |    Checksum    |

+--------+--------+--------+--------+

|           application data (payload) |

+--------------------------------------+

- Length: bytes of UDP segment (header + payload)

- Checksum: detects errors (flipped bits)

### UDP Checksum (Internet Checksum)

Sender:

- Treat segment content (including UDP header fields and IP addresses) as 16-bit integers

- Compute one's complement sum of segment content

- Put checksum value into UDP checksum field

Receiver:

- Compute checksum of received segment

- If computed ≠ checksum field → error detected

- If equal → no error detected (but errors may still exist – weak protection!)

Example (add two 16-bit ints with wraparound):

text

1110 0110 0110 0110  (16-bit int)

1101 0101 0101 0101  (another)

------------------------

1101 1011 1011 1011  (sum, with carry)

1  (wraparound)

1011 1011 1011 1100  (one's complement sum)

0100 0100 0100 0011  (checksum)

Internet checksum is weak compared to CRC, but fast.

# 📖 Final Quick Reference – Key Terms

| Term | Meaning |
| --- | --- |
| ISP | Internet Service Provider |
| IXP | Internet Exchange Point |
| RTT | Round-Trip Time |
| FDM/TDM | Frequency/Time Division Multiplexing |
| HOL | Head-of-Line blocking |
| TLS | Transport Layer Security |
| SMTP | Simple Mail Transfer Protocol |
| IMAP | Internet Mail Access Protocol |
| DNS | Domain Name System |
| CDN | Content Distribution Network |
| UDP | User Datagram Protocol |
| TCP | Transmission Control Protocol |
| rdt | Reliable Data Transfer |
| ACK/NAK | Acknowledgment / Negative Acknowledgment |
| GBN | Go-Back-N |
| SR | Selective Repeat |
| cwnd | Congestion window |
| ssthresh | Slow start threshold |
| AIMD | Additive Increase Multiplicative Decrease |

"The only true wisdom is in knowing you know nothing." – Socrates
"Networks are complex – layering helps us manage that complexity."

End of Computer Networking Notes 📚✅